Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 10 14 23 - PANEL SIGNAGE

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Panel signs.

          - Room-identification signs.

          - Field-applied, vinyl-character signs.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 01 50 00 "Temporary Facilities and Controls" for temporary Project identification signs and for temporary information and directional signs.

          - Section 01 56 39 "Temporary Tree and Plant Protection" for temporary protection-zone signage.

          - [Section 14 21 00 "Electric Traction Elevators"] [Section 14 24 00 "Hydraulic Elevators"] or code-required conveying equipment signage.

          - Section 22 05 53 "Identification for Plumbing Piping and Equipment" for labels, tags, and nameplates for plumbing systems and equipment.

          - Section 23 05 53 "Identification for HVAC Piping and Equipment" for labels, tags, and nameplates for HVAC systems and equipment.

          - Section 26 05 53 "Identification for Electrical Systems" for labels, tags, and nameplates for electrical equipment.

          - Section 26 51 00 "Interior Lighting" for illuminated, self-luminous, and photoluminescent exit sign units.

      - ALLOWANCES

Retain products and work included in this Section that are covered by cash or quantity allowance.  Do not include amounts.  Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the allowance.  Delete this article if all work is done by lump-sum price.

        - Allowances for [signage] [room-identification signs] <Insert item description> are specified in Section 01 21 00 "Allowances."

        - <Insert product or material> [is] [are] part of <Insert name of allowance>.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

        - Accessible:  In accordance with the accessibility standard.

        - Illuminated:  Illuminated by lighting source integrally constructed as part of the sign unit.

      - COORDINATION

Retain this article if signs are mounted on permanent construction with preinstalled anchors or electrical service; delete article if signs are installed using field-drilled anchorage methods and have no electrical service.

Retain first paragraph below for signs that require installation of anchorage devices embedded in permanent construction by other installers.

        - Furnish templates for placement of sign-anchorage devices embedded in permanent construction by other installers.

Retain paragraph below for illuminated signs that require accurate installation of electrical service embedded in permanent construction by other installers.

        - Furnish templates for placement of electrical service embedded in permanent construction by other installers.

      - ACTION SUBMITTALS

        - Product Data:  For each type of product.

        - LEED Submittals (Projects authorized for LEED certification only)

"Product Certificates for Credit MR 5" Subparagraph below applies to LEED-CI.

          - Product Certificates for Credit MR 5:  For products and materials required to comply with requirements for regionally manufactured materials.  Include statement indicating cost for each regionally manufactured material.

First subparagraph below applies to LEED-CI Credit MR 5, Option 1.

            - Include statement indicating location of manufacturer and distance to Project for each regionally manufactured material.

"Product Data for Credit IEQ 4.1" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for adhesives.

          - Product Data for Credit IEQ 4.1:  For adhesives, documentation including printed statement of VOC content.

"Laboratory Test Reports for Credit IEQ 4.1" Subparagraph below applies to LEED for Schools.

          - Laboratory Test Reports for Credit IEQ 4.1:  For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

        - Shop Drawings:  For panel signs.

          - Include fabrication and installation details and attachments to other work.

          - Show sign mounting heights, locations of supplementary supports to be provided by others, and accessories.

          - Show message list, typestyles, graphic elements[, including raised characters and Braille], and layout for each sign at least [half size] <Insert scale>.

Retain two subparagraphs below if signs are electrically powered.

          - Show locations of electrical service connections.

          - Include diagrams for power, signal, and control wiring.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection:  For each type of sign assembly, exposed component, and exposed finish.

Retain subparagraph below if typestyle has not been previously selected.

          - Include representative Samples of available typestyles and graphic symbols.

        - Samples for Verification:  For each type of sign assembly showing all components and with the required finish(es), in manufacturer's standard size unless otherwise indicated and as follows:

Revise list below to suit Project.  Insert specific sign Samples and sizes if required.

          - Panel Signs:  [Full-size Sample] [Not less than 12 inches (300 mm) square, including corner] <Insert size>.

          - Room-Identification Signs:  [Full-size Sample] <Insert size>.

          - Field-Applied, Vinyl-Character Signs:  [Full-size Sample of characters on glass] <Insert requirement>.

          - Variable Component Materials:  [Full-size Sample] [8-inch (200-mm) Sample] <Insert size> of each base material, character (letter, number, and graphic element) in each exposed color and finish not included in Samples above.

          - Exposed Accessories:  [Full-size Sample] [Half-size Sample] <Insert size> of each accessory type.

        - Sign Schedule:  Use same designations specified or indicated on Drawings or in a sign schedule.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

        - Delegated-Design Submittal:  For [signs indicated in "Performance Requirements" Article] <Insert sign designations>.

          - Include structural analysis calculations for signs indicated to comply with design loads; signed and sealed by the qualified professional engineer responsible for their preparation.

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data:  For [Installer] [and] [manufacturer].

        - Sample Warranty:  For special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data:  For signs to include in maintenance manuals.

      - QUALITY ASSURANCE

Retain "Installer Qualifications" Paragraph below to suit Project; revise if Installer qualification is required only for specific sign(s).  Verify availability with sign manufacturer; some manufacturers do not offer installation services, particularly for smaller signs.

        - Installer Qualifications:  [Manufacturer of products] [An entity that employs installers and supervisors who are trained and approved by manufacturer].

      - FIELD CONDITIONS

Retain "Field Measurements" Paragraph below for signs that require installation of anchorage devices, electrical service, or both, embedded in permanent construction by other installers.  Anchorage devices embedded in permanent construction by other installers are uncommon.

        - Field Measurements:  Verify locations of [anchorage devices] [and] [electrical service] embedded in permanent construction by other installers by field measurements before fabrication, and indicate measurements on Shop Drawings.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty:  Manufacturer agrees to repair or replace components of signs that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Deterioration of finishes beyond normal weathering.

            - Deterioration of embedded graphic image.

            - Separation or delamination of sheet materials and components.

Verify available warranties and warranty periods for units and components.

          - Warranty Period:  [Five] <Insert number> years from date of Substantial Completion.

- PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products.  For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

      - PERFORMANCE REQUIREMENTS

        - Accessibility Standard:  Comply with applicable provisions in   Texas Accessibility Standards (TAS) for signs.

      - SIGNS

Retain this article for panel signs consisting of flat or curved surfaces bearing multiple characters.

Copy "Manufacturers," "Basis-of-Design Product," "Panel Sign," and "Room-Identification Sign" paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required.  Use these designations on Drawings to identify each product.

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed.  Retain "Basis-of-Design Product" subparagraphs in remainder of this article if retaining paragraph below.

        - Basis-of-Design Product:  Subject to compliance with requirements, provide plaque made of New Hermes Gravo-Tac material with raised numbers and Grade 2 Braille provided by New Hermess, Inc.; www.newhermes.com or comparable product by one of the following:

Manufacturers listed below use one or more of graphic processes specified.  Revise list to suit Project.

          - Accent Signage Systems, Inc.; www.accentsignage.com.

          - ASI Sign Systems, Inc, InTac System; www;asisignage.com.

          - Signs of Success; www.signsuccess.com

          - Substitutions:  See Section 01 25 00 - Substitution Procedures.

Generally retain "Room-Identification Sign" Paragraph below for a system of small panel signs to designate rooms and spaces in a building.  Generally, these must comply with the accessibility standard for characters, Braille, and placement.

        - Room-Identification Sign <Insert drawing designation>:  Sign with smooth, uniform surfaces; with message and characters having uniform faces, sharp corners, and precisely formed lines and profiles; and as follows:

          - Basis-of-Design Product:  Subject to compliance with requirements, provide 1/16" thick plaque made of New Hermes Gravo-Tac material with 1/32" thick 3/4" tall white raised numbers and Grade 2 Braille provided by New Hermess, Inc.; www.newhermes.com or comparable product by one of the following:.

Available room-identification-sign systems vary with manufacturer; most offer some customization, and many offer materials and processes they claim are unique.  Polymer assembly in "Laminated-Sheet Sign" Subparagraph below is an example of those available for room-identification systems.  Insert additional materials to suit Project; delete subparagraphs not required.  Consult manufacturers for materials suitable for the exposure and required performance.

          - Laminated-Sheet Sign:    New Hermes Gravo-Tac material face sheet with raised graphics laminated to    backing sheet to produce composite sheet.

            - Composite-Sheet Thickness:      3/32".

Retain one of or both "Surface-Applied Graphics" and "Subsurface Graphics" subparagraphs below to suit Project.

            - Surface-Applied Graphics:  Applied Gravo-Tac   .

            - Color(s):    Back Plate:  Taupe #216-214.  Raised Letters:  Bright White #214-114.  Braille to match back plate.

Generally retain "Sign-Panel Perimeter" or "Frame" Subparagraph below; revise if sign panel is partially framed with horizontal or vertical retainers.  Include requirements only to the extent that they are not detailed on Drawings or scheduled; delete others.

          - Sign Perimeter:  Finish edges smooth.

            - Edge Condition:   Square cut   .

            - Corner Condition in Elevation:   Square  .

          - Mounting:  Manufacturer's standard method for substrates indicated  with       .

Retain first option in "Text and Typeface" Subparagraph below unless signs are not required to be accessible.  Retain one or more other options if this information is not fully indicated on Drawings, such as for variable text signs, or if some text is not required to be accessible.

          - Text and Typeface for Room Identification:  Accessible raised characters and Braille.  Century Gothic Bold, 3/4" tall, Bright White.  Braille to match background color.

          - Text and Typeface for Department and Suite Identification:  Accessible raised characters Century Gothic Bold, 5/8" tall, Bright White.  No Braille.

      - ACCESSORIES

        - Fasteners and Anchors:  Manufacturer's standard as required for secure anchorage of signage, noncorrosive and compatible with each material joined, and complying with the following:

          - Use concealed fasteners and anchors unless indicated to be exposed.

Use materials in "Two-Face Tape," "Hook-and-Loop Tape," and "Magnetic Tape" paragraphs below for small signs only; they are suitable for smooth, nonporous surfaces.  Two-face tape is generally 3M brand's "VHB Heavy Duty Mounting Tape."

        - Two-Face Tape:  Manufacturer's standard high-bond, foam-core tape, 0.045 inch (1.14 mm) thick, with adhesive on both sides.

Material in "Hook-and-Loop Tape" Paragraph below is suitable for easily removable signs or sign components.  Hook side of hook-and-loop tape can be used for mounting signs on some fabrics; consult manufacturer for recommendations.

        - Hook-and-Loop Tape:  Manufacturer's standard two-part tape consisting of hooked part on sign back and looped side on mounting surface.

Material in "Magnetic Tape" Paragraph below is suitable for easily removable signs or sign components and for mounting on steel surfaces only.

        - Magnetic Tape:  Manufacturer's standard magnetic tape with adhesive on one side.

      - GENERAL FINISH REQUIREMENTS

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work:  Noticeable variations in same piece are not acceptable.  Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of signage work.

        - Verify that sign-support surfaces are within tolerances to accommodate signs without gaps or irregularities between backs of signs and support surfaces unless otherwise indicated.

Retain first paragraph below if signs are mounted with preinstalled anchors; delete if signs are installed using drilled-in-place-anchor methods.

        - Verify that anchor inserts are correctly sized and located to accommodate signs.

Retain first paragraph below for internally illuminated signs.

        - Verify that electrical service is correctly sized and located to accommodate signs.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

Retain mounting methods in this article that coordinate with mounting requirements in sign-description paragraphs retained in Part 2.

        - General:  Install signs using mounting methods indicated and according to manufacturer's written instructions.

          - Install signs level, plumb, true to line, and at locations and heights indicated, with sign surfaces free of distortion and other defects in appearance.

          - Install signs so they do not protrude or obstruct according to the accessibility standard.

          - Before installation, verify that sign surfaces are clean and free of materials or debris that would impair installation.

          - Corrosion Protection:  Coat concealed surfaces of exterior aluminum in contact with grout, concrete, masonry, wood, or dissimilar metals, with a heavy coat of bituminous paint.

Retain "Room-Identification Signs( and Other Accessible Signage)" Paragraph below if required; revise to suit Project.  Generally, the installation must comply with the accessibility standard.  Delete paragraph if all locations are precisely scheduled or indicated on Drawings.

        - Room-Identification Signs[ and Other Accessible Signage]:  Install in locations on walls [as indicated] [and] [according to accessibility standard] <Insert requirement>.

        - Mounting Methods:

          - Concealed Studs:  Using a template, drill holes in substrate aligning with studs on back of sign.  Remove loose debris from hole and substrate surface.

            - Masonry Substrates:  Fill holes with adhesive.  Leave recess space in hole for displaced adhesive.  Place sign in position and push until flush to surface, embedding studs in holes.  Temporarily support sign in position until adhesive fully sets.

            - Thin or Hollow Surfaces:  Place sign in position and flush to surface, install washers and nuts on studs projecting through opposite side of surface, and tighten.

          - Projecting Studs:  Using a template, drill holes in substrate aligning with studs on back of sign.  Remove loose debris from hole and substrate surface.

            - Masonry Substrates:  Fill holes with adhesive.  Leave recess space in hole for displaced adhesive.  Place spacers on studs, place sign in position, and push until spacers are pinched between sign and substrate, embedding the stud ends in holes.  Temporarily support sign in position until adhesive fully sets.

            - Thin or Hollow Surfaces:  Place spacers on studs, place sign in position with spacers pinched between sign and substrate, and install washers and nuts on stud ends projecting through opposite side of surface, and tighten.

          - Through Fasteners:  Drill holes in substrate using predrilled holes in sign as template.  Countersink holes in sign if required.  Place sign in position and flush to surface.  Install through fasteners and tighten.

          - Brackets:  Remove loose debris from substrate surface and install backbar or bracket supports in position so that signage is correctly located and aligned.

          - Adhesive:  Clean bond-breaking materials from substrate surface and remove loose debris.  Apply linear beads or spots of adhesive symmetrically to back of sign and of suitable quantity to support weight of sign after cure without slippage.  Keep adhesive away from edges to prevent adhesive extrusion as sign is applied and to prevent visibility of cured adhesive at sign edges.  Place sign in position, and push to engage adhesive.  Temporarily support sign in position until adhesive fully sets.

          - Two-Face Tape:  Clean bond-breaking materials from substrate surface and remove loose debris.  Apply tape strips symmetrically to back of sign and of suitable quantity to support weight of sign without slippage.  Keep strips away from edges to prevent visibility at sign edges.  Place sign in position, and push to engage tape adhesive.

Revise "Hook-and-Loop Tape" Subparagraph below if applying both components of tape simultaneously to back of sign and pressing to substrate surface; this facilitates better alignment of tape components, but can result in poorer bond to substrate.

          - Hook-and-Loop Tape:  Clean bond-breaking materials from substrate surface and remove loose debris.  Apply sign component of two-part tape strips symmetrically to back of sign and of suitable quantity to support weight of sign without slippage; push to engage tape adhesive.  Keep tape strips [0.250 inch (6.35 mm)] <Insert dimension> away from edges to prevent visibility at sign edges when sign is initially installed or reinstalled.  Apply substrate component of tape to substrate in locations aligning with tape on back of sign; push and rub well to fully engage tape adhesive to substrate.

          - Magnetic Tape:  Clean bond-breaking materials from substrate surface and remove loose debris.  Apply tape strips symmetrically to back of sign and of suitable quantity to support weight of sign without slippage.  Keep strips away from edges to prevent visibility at sign edges.  Place sign in position.

Retain "Shim-Plate Mounting" Subparagraph below if signs are to be applied to irregular surfaces.  Indicate locations on Drawings or by inserts in sign-description paragraphs retained in Part 2.

          - Shim-Plate Mounting:  Provide 1/8-inch- (3-mm-) thick, concealed aluminum shim plates with predrilled and countersunk holes, at locations indicated, and where other direct mounting methods are impractical.  Attach plate with fasteners and anchors suitable for secure attachment to substrate.  Attach signs to plate using <Insert mounting method> method specified above.

        - Field-Applied, Vinyl-Character Signs:  Clean and dry substrate.  Align sign characters in final position before removing release liner.  Remove release liner in stages, and apply and firmly press characters into final position.  Press from the middle outward to obtain good bond without blisters or fishmouths.  Remove carrier film without disturbing applied vinyl film.

Retain "Signs Mounted on Glass" Paragraph below if applicable.  These can be field-applied, vinyl-characters or other thin signs.

        - Signs Mounted on Glass:  Provide opaque sheet matching sign material and finish onto opposite side of glass to conceal back of sign.

      - ADJUSTING AND CLEANING

        - Remove and replace damaged or deformed signs and signs that do not comply with specified requirements.  Replace signs with damaged or deteriorated finishes or components that cannot be successfully repaired by finish touchup or similar minor repair procedures.

        - Remove temporary protective coverings and strippable films as signs are installed.

        - On completion of installation, clean exposed surfaces of signs according to manufacturer's written instructions, and touch up minor nicks and abrasions in finish.  Maintain signs in a clean condition during construction and protect from damage until acceptance by Owner.

Insert a sign schedule here or on Drawings to list each sign type by drawing designation, text, and quantity, coordinated with Drawings.  Also include sign size, font, colors, and locations to the extent that these items are not shown on Drawings.  See Evaluations for an example of a sign schedule.

END OF SECTION 10 14 23