NAME:

Problem 1:  Definitions    Match the following terms and definition (note: some terms are not defined)

| Terms | Definitions |
| --- | --- |
| 1. Voltage | A sensor which measures temperature as a resistance change. |
| 2. Current | A prefix meaning one millionth |
| 3. Micro | The unit for power |
| 4. Ohm | A battery rating |
| 5. Resistivity | The force driving current through a circuit |
| 6. Watt | Specification of the size of a wire |
| 7. Thermistor | A flow of charges in a wire |
| 8. kilowatthour | The unit of resistance |
| 9. AWG # 12 | The unit of charge |
| 10. Ampere-Hour | The unit for measuring home energy consumption. |
| 11. Coulomb |  |
| 12. Electron |  |
| 13. Milli |  |

Problem 2:

A 9-volt battery is used for a small AM/FM radio.  The radio draws a current of 160 mA.  The capacity of the battery is 0.5 Ah.  Answer the following questions:

- How many hours will the battery last?

- How much power is delivered by the battery?

- How many joules of energy does the radio consume is 20 minutes?

Problem 3:

Consider an old Wyoming telegraph line strung on poles over a distance of 30 miles.  The line is made of copper wire with a diameter of 0.125 inches.  The resistivity is  = 10.37 -CM/ft

- What is the resistance of the copper wire?

b.  The copper temperature coefficient is given by,  = 0.00393  at 20oC.   Assuming part a. was at 20oC, what will the wire resistance be on a summer day when the temperature is 110oF?

Problem 4:	A ten volt battery is connected to a 5 k resistor.

- What current flows through the resistor?

- How much power is dissipated by the resistor?

- Draw a schematic of the circuit

Problem 5:  Given the following resistor:

- By color code what is its resistance?

- b.    What minimum and maximum actual resistance could it have?