Problem 1:  Circle the best answer for the following questions:

| a. The current carriers in a metal are: | 1. electrons 2. protons 3. ions 4. atoms | b.A resistor color code of red-red-red means a resistance of: | 1. 222     |
| --- | --- | --- | --- |
| c. A kilowatthour (kwh) is a unit of: | 1. voltage 2. current 3. power 4. energy | d. How do you write 0.000000345 A using the micro? | 1. 345 A 2. 34.5 A 3. 0.345 A 4. 3.45 A |
| f. Which material would have the least reluctance to magnetic flux? | 1. iron 2. copper 3. wood 4. carbon | g. The unit of inductance is the: | 1. Henry 2. Farad 3. Joule 4. Weber |
| i. As the separation between capacitor plates gets smaller the capacity: | 1. stays the same 2. gets larger 4. gets smaller 5. gets going | j. How long will a battery with a 1.2 Ah rating deliver 24 mA? | 1. 50 minutes 2. 28.8 hours 3. 50 hours 4. 20 hours |

Problem 2:  Use Delta-Y or Y-Delta Transformation to find the resistance between terminals a and b of the following circuit:  (resistance  in Ohms)

Problem 3:  Given the following circuit:

(resistance in Ohms)

(a)  Determine the Thevenin Equivalent Circuit, draw the circuit and show the values and polarities.

(b)  Determine the Norton Equivalent circuit using the Thevenin.

Problem 4:  A simple capacitor is made of square parallel plates with a mica separator as shown.  The plate separation is 0.15 mm and the relative permittivity of mica is r = 5.  Find the square dimension, x, so the capacity will be 1.6 F.

Problem 5:  In the following circuit the switch closes at t = 0.

a.  What is the time constant?

b.  At what time will the voltage across the capacitor reach 15.5 volts?

c.  Sketch the current and capacitor voltage as a function of time

Problem 6:  Use MESH to find the equations which will solve the following circuit.  Do not solve!   Just determine the equations.

Problem 7:  What current is required to have a flux of 6 x 10-4 Wb?

Problem 8:  In the following circuit the switch has been in position a for a long, long time.  At t = 0 the switch is moved to position b, now:

(a)  What is the time constant ?

(b)  What is the inductor voltage after 1 ms?